MAKİNA MÜHENDİSLİĞİ BÖLÜMÜ ME 492 ENGINEERING PROJECT

PROJECT ANNOUNCEMENT

Spring 2026

Title: Design and Experimental Evaluation of a High-Stability Robotic Manipulator for Precision Tasks

Position Ref: CPC_ARM_S26

Type: Group

Status: New

Seats: 2 students

Project Description:

In this project, a robotic manipulator will be designed, analyzed, manufactured, and experimentally evaluated with an emphasis on mechanical stiffness, transmission quality, and dynamic stability. The objective is to develop a multi-degree-of-freedom arm capable of precise positioning and steady interaction with its environment. Students will begin with a literature survey on robotic arm mechanisms, transmission systems, and vibration sources in manipulators. Based on the identified requirements, conceptual designs will be generated and refined into a detailed mechanical design. Kinematic and structural models will be developed to predict workspace, joint loading, deflections, and dynamic behavior. A physical prototype will be constructed, and experimental tests will be conducted to evaluate positioning accuracy, oscillation behavior, and load-handling capability. Analytical and simulation results will be compared with experimental observations.

System Requirements and Design Constraints:

- Minimum number of degrees of freedom: 4 DOF

- Minimum horizontal reach from base: 600 mm

- Minimum vertical reach from base: 800 mm

- Minimum payload capacity at maximum reach: 1 kg

- Minimum continuous interaction force at end-effector: 10 N

- End-effector positional repeatability: ±2 mm

- Visible oscillations after motion stop must decay within 1 s

- Transmission systems must be low-backlash and high-stiffness

  - High-backlash hobby-grade geartrains are not acceptable without mitigation

- Structural deflection under load must be modeled and experimentally evaluated

Students must justify choices of transmission type (e.g. belt, cable, cycloidal, harmonic-like mechanisms), joint layout, and structural geometry using mechanical analysis and/or simulation.

Work Description:

- Literature survey on robotic arm mechanisms and transmissions

- Conceptual and detailed mechanical design (CAD)

- Kinematic and structural modeling of the manipulator

- Manufacturing or assembly of the robotic arm

- Experimental evaluation of accuracy, steadiness, and load capability

- Comparison of analytical, simulation, and experimental results

Required Qualifications and Skills:

- Basic knowledge of mechanics, dynamics, and machine elements

- Ability to perform mechanical design using CAD software

- Familiarity with kinematics and basic structural analysis

- Carefulness in manufacturing and experimental measurements

- The project will be carried out on campus

Related UN Sustainable Development Goals:

- No.9: Industry, Innovation and Infrastructure

Difficulty:		high (X)	medium ( )		low ( )

Time commitment: 	high (X)	medium ( )		low ( )

Estimated expense:	To be determined